Provisional Patent Application: LoopGuard Device Authentication System with Physical Integrity Reset Mechanism

Applicant: Bobby Simpson
Filed: 6/14/25

### Title

Multi-Factor Device Authentication and Tamper-Responsive Loop Verification with Physical Integrity Sensors

### Technical Field

This disclosure relates to verification systems for public-facing physical devices, integrating cryptographic time-based loop authentication with hardware-level tamper detection and conditional loop invalidation.

### Background

Publicly accessible devices—such as ATMs, gas pumps, lockers, kiosks, and vending machines—are subject to impersonation and tampering. While QR-code and TOTP authentication schemes can prove device identity, physical integrity remains a weak link. Traditional tamper switches offer only binary responses, often logged silently or ignored.

### Summary

The disclosed invention combines a TOTP-based loop validation system with a hardware-triggered integrity reset layer that zeros cryptographic secrets or visibly suspends trust indicators when physical intrusion is detected. This ensures not only that a device was valid, but that it remains physically trusted.

### Detailed Description

- Device Loop Initialization:

  - Device receives a cryptographically signed seed from a trusted entity (e.g., manufacturer, brand).

  - The seed is stored securely and used to generate time-synchronized identifiers (e.g., short codes, QR payloads).

- User Verification Interface:

  - The device displays a QR code and a short alphanumeric or numeric challenge code.

  - The user scans the QR or enters the code into a mobile app or web interface.

  - The client app validates the code locally (offline) or via an online API call.

- Tamper-Responsive Integrity System:

  - The device includes one or more hardware sensors configured to detect compromise conditions:

    - Light exposure (e.g., via photodiode or light sensor)

    - Magnetic presence loss (via hall-effect or reed switch)

    - Pressure or tension changes (e.g., latch or button displacement)

    - Accelerometer or shock-based anomalies

    - EM field detection (attempted scanning or probing)

  - Upon trigger of any sensor:

    - The device invalidates its loop seed (zeroed or locked)

    - Device display changes to a non-trust state

    - Loop codes are suspended or scrambled

    - Reset requires credentialed technician access or reseeding event

- Security Features:

  - All sensors are monitored in real-time

  - Multiple conditions may be combined (AND/OR logic)

  - Tamper logs may be stored and encrypted locally

### Claims

- A physical device authentication system comprising:

  - a TOTP-based code generation module;

  - a physical integrity monitoring subsystem;

  - wherein tampering triggers a reset or invalidation of loop credentials.

- The system of claim 1, wherein the physical integrity monitoring subsystem includes one or more of: light sensor, magnetic switch, pressure switch, accelerometer, or EM detection circuit.

- The system of claim 1 or 2, wherein tamper detection results in the deletion, locking, or masking of a cryptographic seed.

- The system of claim 1 or 2, wherein user devices verify loop integrity via QR code, short code, or NFC scan.

- The system of claim 1 or 2, wherein loop status is shown via glyph, color display, or LED indicator.

- The system of claim 1 or 2, wherein verification may occur locally on the client or via a cloud-based API.

- The system of claim 1 or 2, wherein a tamper reset requires technician authentication.

- The system of claim 1 or 2, wherein successful loop validation confirms both identity and physical integrity.

### End of Provisional Draft