# Dynamic QR Wearable — Spec & Prototype Plan

Purpose: A wearable (pendant/badge/necklace) that displays a dynamic QR containing a cryptographic attestation for the current second. Cameras and video capture this QR; verifiers can cryptographically validate presence at a particular moment to raise the cost of deepfake-forgery.

## 1. Executive summary

A small wearable display (pendant/badge/necklace) shows a per-second QR encoding {version, device_id, unix_ts, signature} where signature = Sign(privKey, unix_ts) using an on-device secure element. The QR updates regularly (configurable, e.g., 1 Hz). A verifier scans the QR or extracts it from recorded video and verifies the signature against a public key registry to attest the wearable was physically present and cooperating at time ts.

Benefits: high visibility, simple verification (QR), realistic battery/size tradeoffs, easy integration with existing camera/video workflows.

## 2. Goals & success metrics

- Goals

  - Produce a prototype pendant/badge that reliably renders a per-second cryptographic QR readable by smartphones and by camera video frames.

  - Provide a verifier reference (server + mobile/web app) that validates attestation within a small time window.

  - Demonstrate robustness against typical video compression and lighting conditions.

- Success metrics (prototype)

  - QR readable by smartphone at 1–3m in standard indoor lighting.

  - Video-decoded QR bit error rate (BER) < 1% across tested camera models at 1–3m when updating at 1 Hz.

  - Signing & verification latency < 200 ms on-device (secure element signing time + QR render).

  - Battery life: 8+ hours with 1 Hz updates (target for LiPo 150–300mAh pendant).

## 3. Threat model & limitations

- Assumptions

  - Adversary does not have access to the wearable’s private key (secure element prevents export).

  - Verifier has access to the device’s public key or a trusted registry mapping device_id → pubkey.

- Attacker capabilities considered

  - Recording, editing, and splicing video; attempting to paste a valid QR into forged video.

  - Attempting to reproduce QR patterns visually or synthesize them via generative tools.

- Mitigations

  - Short acceptance window around ts (configurable, e.g., ±5–30 s) reduces replay opportunity.

  - Use asymmetric signatures (Ed25519) stored in a secure element to require private-key access to forge.

  - Optional: hybrid mode where verifier can supply a nonce via BLE/screen when extremely high assurance is needed.

- Limitations

  - If private key is compromised, the attacker can sign arbitrarily (revocation & registry required).

  - Offline timestamp-only signing cannot prevent a determined paste-replay attack within the acceptance window.

## 4. Data format & QR encoding

Recommended binary layout (compact): CBOR array: [v, id, ts, sig] - v (1 byte) — protocol version. - id (8 bytes) — device id (truncated SHA-256 of public key). - ts (4 bytes) — unix epoch seconds (unsigned). - sig (64 bytes) — Ed25519 signature of the canonical input (e.g., id || ts || v).

Canonical signing input: Sign(privKey, v || id || ts) using the device’s private key.

QR modes: use binary mode for QR to minimize size. Render as PNG / frame buffer for the device display. Keep QR image at least 300×300 px for robust scanning by camera/video.

Example JSON (human readable):

{"v":1,"id":"A1B2C3D4","ts":1700000000,"sig":"base64url(...)"}

(Use CBOR/binary in production.)

## 5. Cryptography & key management

- On-device: Private key stored and used inside a secure element (recommend ATECC608A or similar). Device should not expose private key.

- Key type: Ed25519 recommended for compact, fast signatures.

- Public registry: Map id → pubkey and optionally status (active/revoked). Registry can be centralized or federated; verifiers must fetch it periodically or cache entries.

- Revocation: Provide an API to revoke device IDs and propagate revocations to verifiers.

- Time sync: Device maintains RTC; provisioning should set initial time; occasional BLE sync recommended.

## 6. Hardware recommendations (prototype)

- MCU: nRF52840 (BLE + low-power), or STM32 + BLE module.

- Secure element: ATECC608A (supports Ed25519-like signing), or equivalent secure element.

- Display: Choose based on power vs. refresh tradeoff:

  - OLED (128×128 or larger) — fast per-second refresh, good contrast, moderate power draw. Recommended for per-second QR.

  - e-Paper (partial refresh capable) — excellent battery life, slower updates. Use if you plan updates every 2+ seconds and battery is the priority.

- Battery: 150–300 mAh LiPo for pendant; coin cell only for very low update rates.

- Haptics / LEDs: optional single LED or vibration motor for user feedback on successful signing.

- Enclosure: pendant/badge window area 25–40 mm square for QR visibility.

## 7. Firmware — core flow

- Boot and initialize secure element and RTC.

- Every interval seconds (configurable; default 1s):

  - Read ts = unix_seconds() from RTC.

  - Build canonical input payload = v || id || ts.

  - Request sig = SecureElement.Sign(payload).

  - Build CBOR [v, id, ts, sig] and encode to QR binary mode.

  - Render QR to framebuffer and refresh display.

  - Optional: short haptic/LED pulse to indicate update.

- BLE provisioning service for pairing, public key retrieval, time sync, firmware OTA.

Performance target: signing + render should complete < 200 ms to keep UI snappy.

## 8. Verifier — reference design

- Input sources: live camera scan (mobile app), uploaded video file (server or app extracts frames), or manual scan by user.

- Steps:

  - Scan QR or extract QR image from video frame(s).

  - Decode CBOR -> {v, id, ts, sig}.

  - Lookup pubkey for id in registry; if not found reject or query registry endpoint.

  - Verify signature: Verify(pubkey, v||id||ts, sig).

  - Check abs(now - ts) <= window and that id is not revoked.

  - Accept attestation: log id, ts, verifier id, and video metadata for audit.

- Video extraction notes: prefer to find the highest-quality frame(s) that capture the QR. If multiple frames available, decode across them to reduce BER.

## 9. Testing & evaluation plan

- Display readability tests

  - Test reading QR from live smartphone at distances (0.5m, 1m, 2m, 3m) and angles (±45°). Record success rates.

- Video robustness tests

  - Record 1 Hz updates in typical indoor lighting on multiple phones (iPhone 12/13/14, Android midrange and flagship). Compress with H.264/H.265 at typical streaming bitrates. Extract frames and attempt decode; measure BER/decoding success.

- Latency & battery tests

  - Measure signing + render time; run continuous updates and log battery depletion to estimate operational life.

- Replay & attack simulation

  - Attempt to record a valid QR and paste it into forged video to see how acceptance window and other heuristics behave.

- Edge-case tests

  - Extreme low light, strong backlight, motion blur, camera auto-exposure blending (HDR), and various video codecs.

## 10. Bill of Materials (prototype)

- nRF52840 dev module (or equivalent MCU)

- ATECC608A secure element (or dev breakout)

- 1.3” OLED 128×128 (or as chosen)

- 150–300 mAh LiPo battery + charger

- Vibration motor / LED (optional)

- Small PCB and pendant enclosure

Estimated prototype part cost (1–10 units): $60–150 depending on components & enclosure.

## 11. Implementation milestones (suggested)

- Week 0–1 — Design & procurement: finalize display choice, order dev boards and components.

- Week 2–4 — Firmware & QR pipeline: implement signing flow with secure element and generate QR bitmaps off-device first, then on-device QR generation.

- Week 4–6 — Prototype enclosure & integration: build pendant, test readability and battery life.

- Week 6–8 — Verifier & server: implement registry, verifier reference app (mobile/web) and server endpoints for registry & revocation.

- Week 8–10 — Testing & hardening: run video robustness tests, attack simulations, adjust symbol cadence and timing.

Adjust schedule to team resources.

## 12. Open questions & optional enhancements

- Nonce mode: Should we implement BLE nonce-request mode for interactive verification (higher assurance)? Useful for high-security contexts.

- Privacy mapping: How should id map to user identity? Allow opt-in linking to personal profiles, or use anonymous id with opt-in attestation service.

- Multi-factor attestation: Combine QR visual attestation with BLE-signed telemetry (adds strong assurance but requires verifier to have access to BLE logs).

- Revocation & provisioning UX: How will we enroll devices, rotate keys, and revoke compromised devices in the field?

## 13. Next deliverables I can produce for dev

- Binary CBOR byte layout + test vectors.

- Reference QR renderer (Python/Node) that takes a private key + ts and produces a PNG QR.

- Example verifier code (Node/Python) that verifies payloads and checks registry.

- Minimal firmware pseudo-spec for integrating secure element + QR rendering.

Tell me which of the above you want first and I will prepare it.